This study is about a tactile user interface designed to help blind and visually impaired users achieve better situational awareness in an autonomous vehicle.

The tactile user interface can consist of up to seven modules. Each module is responsible for representing exactly one piece of information. Below is a list matching each module number to the specific information it represents:

0 = Orientation at destination

1 = Exit direction & Ground condition at Exit

2 = Obstacles at Exit

3 = distance/time and relationship to the destination

4 = reason for the vehicle stopping

5 = other traffic members

6 = future turn & Upcoming manuevers

These modules can be mounted on a board measuring 5 by 3 squares (x/y format). Information about the current situation is conveyed through linear, vertical, or rotational movements. Users can perceive this information by feeling the three-dimensional elements with different shapes attached to each module.

Each module can also vibrate when the corresponding situation changes. In addition, every module features a button that provides auditory feedback with additional information about the specific situation. The modules themselves are black, while the elements on the modules are colored.

Below is a list showing which information is represented by each module, along with the corresponding functions:

| Number | Module | Rotation | Vibration | Vertical Extension | Linear Extension |
| --- | --- | --- | --- | --- | --- |
| 0 | Orientation at Destination | Top element points into the direction of the final destination in regards to the AV and follows it | Module vibrates for 4 seconds when new information gets presented | Module extends when AV reaches the destination | Vertical Orientation: Linear position reflects the position of the final destination (the middle of linear extension is the position of the AV)" |
| 1 | Exit direction & Ground condition at Exit | Top element points into the direction of the Exit direction | Module vibrates for 4 seconds when new information gets presented | Module extends when AV reaches the destination | Horizontal Orientation: Element moves left or right depending on the exit direction |
| 2 | Obstacles at Exit | Top element points at the obstacle | Module vibrates for 4 seconds when new information gets presented | Module extends when AV reaches the destination & there is a obstacle at the destination | Vertical Orientation: Linear position reflects the position of the Obstacle (the middle of linear extension is the position of the AV) |
| 3 | distance/time and relationship to the destination | Top element points into the direction of the final destination in regards to the AV and follows it | Module vibrates for 4 seconds when destination is reached | Module drives in when destination is reached | Horizontal Orientation: Linear movement acts as a progress bar for the estimated time until reaching the destination |
| 4 | reason for the vehicle stopping | 1. Top element points into the direction of the the reason for the stop and follows him (e.g. for pedesstrian crossing) else it does nothing 2. Top element functions as a clock and gives estimated time for the stop | Module vibrates for 4 seconds when new information gets presented | Module extends when AV stops | Horizontal Orientation: Linear movement acts as a progress bar for the estimated stopping time |
| 5 | other traffic members | Top element points into the direction of the traffic member and follows him | Module vibrates for 4 seconds when new information gets presented | Module extends when there are other traffic members close to the AV | Vertical Orientation: Postion reflects the position of the other traffic member (the middle of linear extension is the position of the AV) |
| 6 | future turn & Upcoming manuevers | Top element points into the direction of the upcomming curve | Module vibrates for 4 seconds when new information gets presented | Module extends when there is an upcomming curve | Vertical Orientation: Distance between element and top boundary signals the distance to the upcomming curve |

The parameters that can be modified are:

(local) Position of the modules on the board:
Tuple (x, y)  [x: int 0–4 y: int 0–2]

(local) Used traffic information:
Boolean value [0, 1]  [Orientation at destination, Exit direction & Ground condition at exit, Obstacles at exit, Distance/time and relation to the destination, Reason for the vehicle stopping, Other traffic participants, Future turns & Upcoming maneuvers]

(global) Color of the module elements:
Triple tuple (R, G, B)  [RGB: int 0–255]

(local) Shape of the module elements:
Categorical object type  SHAPES = ["Circle", "Triangle", "Square", "Hexagon", "Arrow"]

(global) Tactile feedback:
Type of tactile feedback for the button used on the elements → Discrete object type  [int 0–2]
0 = No tactile feedback [MX Black Clear Top]
1 = Tactile feedback [MX Grey]
2 = Tactile and clicky feedback [MX Green]

(global) Servo speed:
Speed at which the elements on the modules move  [int: 100°/s – 300°/s]

(global) Vibration intensity:
Intensity with which a module vibrates when conveying information  [int: 65% – 100%]

(global) Vibration pattern:
Pattern used for module vibration:

- On-Time (PD): [int: 5 ms – 200 ms]

- Off-Time (IPI): [int: 10 ms – 500 ms]

(local) Functions used per module:

- use_rotation: [bool]

- use_vibration: [bool]

- use_horizontal_movement: [bool]

(global) Sound output speed:
Speed at which verbal information is delivered  [int: 100 – 250 wpm]

Now, based on this description, decide on a configuration of these parameters that will provide the highest situational awareness, the lowest mental workload, and the best look and feel for the test subject described below.

Keep in mind that you will likely not be able to use all modules, since only one module can be placed on each field, and modules that use linear movement (use_horizontal_movement = large) require two adjacent fields.

You can also find below a list indicating whether a module (identified by its module number) requires one additional space in the x or y direction: (0 = y, 1 = x, 2 = y, 3 = x, 4 = x, 5 = y, 6 = y).

Test Subject (Description based on results from vision tests):

- Amsler Test:

  - The central dot is not visible

  - Presence of “holes” or gray veils

  - Dark or blurry areas

  - Squares appear in different sizes

  - Bent or distorted lines

- Ishihara Color Test:

  - Red-green color deficiency (Deuteranomaly/Protanomaly)

  - Color blindness (stronger form, Dichromasy)

- Snellen Chart Test:

  - 20/40 visual acuity

- Pelli-Robson Chart:

  - logCS = 1.65

- Completely blind.

Provide me with an assignment of these parameters in the following JSON format within their respective defined value ranges (only for the parameters that are also included in the example JSON file). Maintain the JSON format exactly ("extension_length": 8.5 is a fixed value)!

Note that the global parameters are the same for all modules. This means that the RGB colors, tactile feedback, servo speed, vibration intensity, vibration pattern, and sound output speed have the same values for all modules.

Provide all additional parameters that are not included in the JSON file as a separate list (not in JSON format): [Used Traffic Information | Position of the modules on the board | Shape of the module elements | Tactile feedback].

JSON Format:

{

"extension_length": 8.5,

"servo_speed": 200,

"sound_output_speed": 125,

"use_module_0": true,

"large_0": false,

"r_0": 208,

"g_0": 249,

"b_0": 118,

"use_vibration_0": false,

"vibration_intensity_0": 0,

"vibration_on_time_0": 0,

"vibration_off_time_0": 0,

"use_rotation_0": true,

"use_module_1": true,

"large_1": false,

"r_1": 236,

"g_1": 138,

"b_1": 210,

"use_vibration_1": true,

"vibration_intensity_1": 71,

"vibration_on_time_1": 110,

"vibration_off_time_1": 396,

"use_rotation_1": false,

"use_module_2": true,

"large_2": false,

"r_2": 30,

"g_2": 32,

"b_2": 211,

"use_vibration_2": false,

"vibration_intensity_2": 0,

"vibration_on_time_2": 0,

"vibration_off_time_2": 0,

"use_rotation_2": true,

"use_module_3": true,

"large_3": true,

"r_3": 147,

"g_3": 103,

"b_3": 104,

"use_vibration_3": false,

"vibration_intensity_3": 0,

"vibration_on_time_3": 0,

"vibration_off_time_3": 0,

"use_rotation_3": false,

"use_module_4": true,

"large_4": false,

"r_4": 37,

"g_4": 188,

"b_4": 82,

"use_vibration_4": false,

"vibration_intensity_4": 0,

"vibration_on_time_4": 0,

"vibration_off_time_4": 0,

"use_rotation_4": true,

"use_module_5": true,

"large_5": true,

"r_5": 160,

"g_5": 96,

"b_5": 121,

"use_vibration_5": false,

"vibration_intensity_5": 0,

"vibration_on_time_5": 0,

"vibration_off_time_5": 0,

"use_rotation_5": true,

"use_module_6": false,

"large_6": false,

"r_6": 0,

"g_6": 0,

"b_6": 0,

"use_vibration_6": false,

"vibration_intensity_6": 0,

"vibration_on_time_6": 0,

"vibration_off_time_6": 0,

"use_rotation_6": false

}